# COMMISSION ACTION MATRIX 
Accessibility (access) CODE ADVISORY COMMITTEE

## 2025 california building code and 2025 california existing building code, TITLE 24, PARTs 2 and 10, 2024 international building code (IBC) and 2024 international existing building code (IeBC)
AGENCY: division of the state architect, DSA-AC 01/24

### LEGEND:

CAC Actions: Approve, Disapprove, Approve as Amended, Further Study Required

Agency Responses: Accept, Disagree, Withdraw

CBSC Actions: Approve, Disapprove, Approve as Amended, Further Study Required

Matrix Paper Color (for commission action only): GREEN = uncontested items, YELLOW = challenged items, SALMON = withdrawn, no action required

If using assistive technology, please adjust your settings to recognize underline, strikeout, italic and ellipsis.

- Model Code language appears upright

- Existing California amendments appear in italic

- Amended model code or new California amendments appear underlined & italic

- Repealed model code language appears upright and in strikeout

- Repealed California amendments appear in italic and strikeout

- Ellipses (…) indicate existing text remains unchanged

### 2025 CALIFORNIA BUILDING CODE
ITEMS 1 through 19

#### ITEM 1, Chapter 2, Section 202 - Definitions

Propose to amend Section 202 as listed below.

| DSA-AC 01/24 ITEM 1 | Code Section | CAC Action | Agency Response | Public Comments | Annotations | CBSC Action |
| --- | --- | --- | --- | --- | --- | --- |
| - 1-1 | Multi-Bedroom Housing Unit | Approve | Accept |  | Modify and change the term “toilet and bathing room” for “bathroom.” |  |

#### ITEM 2, Chapter 11B, accessibility to public buildings, public accommodations, commercial buildings and public housing, Section 11B-213.2 Toilet rooms and bathing rooms

Propose to amend Sections in Chapter 11B from the 2022 CBC into the 2025 CBC as listed below.

| DSA-AC 01/24 ITEM 2 | Code Section | CAC Action | Agency Response | Public Comments | Annotations | CBSC Action |
| --- | --- | --- | --- | --- | --- | --- |
| - 2-1 | 11B-213.2 Toilet rooms and bathing rooms. | Approve | Accept |  | Propose to remove the language in exception 5 to eliminate duplicative language. |  |

#### ITEM 3, Chapter 11B, Section 11B-224 Transient lodging guest rooms, housing at a place of education and social service center establishments

Propose to amend Sections in Chapter 11B from the 2022 CBC into the 2025 CBC as listed below.

| DSA-AC 01/24 ITEM 3 | Code Section | CAC Action | Agency Response | Public Comments | Annotations | CBSC Action |
| --- | --- | --- | --- | --- | --- | --- |
| - 3-1 | 11B-224.1 General. | Further Study | Accept |  | Propose to amend this section by adding “housing at a place of education for undergraduate students” to appropriately determine the required scope for these facilities. CAC: 9 Point Criteria #6 Text “housing at a place of education for undergraduate students” incorrectly eliminates the applicability of the requirement for other types of housing at a place of education. After CAC: Modified amendment to remove phrase “for undergraduate students”. |  |
| - 3-2 | 11B-224.1.2 Guest room doors and doorways. | Further Study | Accept |  | Propose to amend this section by adding “transient lodging facilities.” To distinguish from “housing at a place of education…” CAC: 9 Point Criteria #6 No suggested amendment for this section. After CAC: No change to existing amendment as the CAC comment applies to Items 3-1 and 3-4 only. |  |
| - 3-3 | 11B-224.1.4 Guest room toilet and bathing rooms. | Further Study | Accept |  | Propose to amend this section by adding “transient lodging facilities.” To distinguish from “housing at a place of education…” CAC: 9 Point Criteria #6 No suggested amendment for this section. After CAC: No change to existing amendment as the CAC comment applies to Items 3-1 and 3-4 only. |  |
| - 3-4 | 11B-224.2 Guest rooms with mobility features. | Further Study | Accept |  | Propose to amend this section by adding: “housing at a place of education …” and “transient lodging facilities” to provide applicable scoping. CAC: 9 Point Criteria #6 Text “housing at a place of education for undergraduate students” incorrectly eliminates the applicability of the requirement for other types of housing at a place of education. After CAC: Modified amendment to remove phrase “for undergraduate students”. |  |

#### ITEM 4, Chapter 11B, section 11B-228.3.2 minimum number

Propose to amend Sections in Chapter 11B from the 2022 CBC into the 2025 CBC as listed below.

| DSA-AC 01/24 ITEM 4 | Code Section | CAC Action | Agency Response | Public Comments | Annotations | CBSC Action |
| --- | --- | --- | --- | --- | --- | --- |
| - 4-1 | 11B-228.3.2 Minimum number. | Approve | Accept |  | Propose to amend the exceptions to provide clarity. |  |

#### ITEM 5, Chapter 11B, section 11B-233.3 public housing facilities

Propose to amend Sections in Chapter 11B from the 2022 CBC into the 2025 CBC as listed below.

| DSA-AC 01/24 ITEM 5 | Code Section | CAC Action | Agency Response | Public Comments | Annotations | CBSC Action |
| --- | --- | --- | --- | --- | --- | --- |
| - 5-1 | 11B-233.3 Public housing facilities. | Approve | Accept |  | Propose to amend section to add a reference pointer directing code users to Section 11B-809. |  |
| - 5-2 | 11B-233.3.1.1 Residential dwelling units with mobility features. | Approve | Accept |  | Propose to amend section to correct reference pointer. |  |
| - 5-3 | 11B-233.3.1.2 Residential dwelling units with adaptable features | Approve | Accept |  | Propose to amend section to add “shall” to note that code user must comply with this section and also remove reference pointer for clarity. |  |

#### ITEM 6, Chapter 11B, Section 11B-233.3.1.2.6.1 Single building with one common (lobby) entrance

Propose to amend Sections in Chapter 11B from the 2022 CBC into the 2025 CBC as listed below.

| DSA-AC 01/24 ITEM 6 | Code Section | CAC Action | Agency Response | Public Comments | Annotations | CBSC Action |
| --- | --- | --- | --- | --- | --- | --- |
| - 6-1 | 11B-233.3.1.2.6.1 Single building with one common (lobby) entrance. | Approve | Accept |  | Propose to provide the correct reference the unusual characteristic test of site impracticality. |  |

#### ITEM 7, Chapter 11B, section 11B-233.3.1.2.6.5 additional requirements

Propose to amend Sections in Chapter 11B from the 2022 CBC into the 2025 CBC as listed below.

| DSA-AC 01/24 ITEM 7 | Code Section | CAC Action | Agency Response | Public Comments | Annotations | CBSC Action |
| --- | --- | --- | --- | --- | --- | --- |
| - 7-1 | 11B-233.3.1.2.6.5 Additional requirements. | Approve | Accept |  | Propose to provide additional information regarding the applicability of the section with new text language and correct some referenced pointers. |  |

#### ITEM 8, Chapter 11B, section 11b-603.6 guest room toilet and bathing rooms

Propose to amend Sections in Chapter 11B from the 2022 CBC into the 2025 CBC as listed below.

| DSA-AC 01/24 ITEM 8 | Code Section | CAC Action | Agency Response | Public Comments | Annotations | CBSC Action |
| --- | --- | --- | --- | --- | --- | --- |
| - 8-1 | 11B-603.6 Guest room toilet and bathing rooms. | Approve | Accept |  | Propose to clarify a California specific requirement applicable to transient lodging facilities only. |  |

#### ITEM 9, Chapter 11B, section 11b-705.1.2.5 blended transitions

Propose to amend Sections in Chapter 11B from the 2022 CBC into the 2025 CBC as listed below.

| DSA-AC 01/24 ITEM 9 | Code Section | CAC Action | Agency Response | Public Comments | Annotations | CBSC Action |
| --- | --- | --- | --- | --- | --- | --- |
| - 9-1 | 11B-705.1.2.5 Blended transitions. | Approve | Accept |  | Propose to clarify requirements for detectable warning surfaces at blended transitions. |  |

#### ITEM 10, Chapter 11B, section 11b-806.2.3.1 personal lift device floor space

Propose to amend Sections in Chapter 11B from the 2022 CBC into the 2025 CBC as listed below.

| DSA-AC 01/24 ITEM 10 | Code Section | CAC Action | Agency Response | Public Comments | Annotations | CBSC Action |
| --- | --- | --- | --- | --- | --- | --- |
| - 10-1 | 11B-806.2.3.1 Personal lift device floor space. | Approve | Accept |  | Propose to amend the technical requirements to clarify that the requirement is applicable to transient lodging. |  |

#### ITEM 11, Chapter 11B, section 11b-806.2.4.1 vanity counter top space

Propose to amend Sections in Chapter 11B from the 2022 CBC into the 2025 CBC as listed below.

| DSA-AC 01/24 ITEM 11 | Code Section | CAC Action | Agency Response | Public Comments | Annotations | CBSC Action |
| --- | --- | --- | --- | --- | --- | --- |
| - 11-1 | 11B-806.2.4.1 Vanity counter top space. | Approve | Accept |  | Propose to amend this section for transient lodging facilities and housing at a place of education where the terms for “bathroom” and “toilet or bathing rooms” denote applicability in both type of facilities. |  |

#### ITEM 12, Chapter 11B, Section 11B-809 Residential dwelling units

Propose to amend Sections in Chapter 11B from the 2022 CBC into the 2025 CBC as listed below.

| DSA-AC 01/24 ITEM 12 | Code Section | CAC Action | Agency Response | Public Comments | Annotations | CBSC Action |
| --- | --- | --- | --- | --- | --- | --- |
| - 12-1 | 11B-809.1 General | Further Study | Accept |  | Propose to amend section providing new referenced section numbers. CAC: 9 Point Criteria #6 No suggested amendment for this section. After CAC: No change to existing amendment. |  |
| - 12-2 | 11B-809.1.1 Door signal devices. | Further Study | Disagree |  | Propose to add new section to clarify requirements for door signal devices in buildings with 3 or more dwelling units. CAC: 9 Point Criteria #6 Provide additional clarity as terms are not defined in the code. After CAC: No change to existing amendment as terms are defined in standard collegiate dictionary and mirrors Section 11B-809.8.4. |  |
| - 12-3 | 11B-809.1.2 Viewing devices. | Further Study | Disagree |  | Propose to add new section to clarify the requirements for viewing devices, to include a lower viewing device when devices are provided. CAC: 9 Point Criteria #6 Clarify height of viewing device and minimum range of view, begin sentence with “each viewing device”. After CAC: DSA revised ET to clarify height requirement but declined to amend language for the range of view. |  |
| - 12-4 | 11B-809.1.3 Washing machines and clothes dryers. | Further Study | Accept |  | Propose to add new section to provide a requirement for washer and dryers in residential dwelling units with mobility features. CAC: 9 Point Criteria #6 Clarify requirements in regard to equivalency for all residential units. After CAC: Language amended according to CAC comments. |  |

#### ITEM 13, Chapter 11B, section 11b-809.6 residential dwelling units with adaptable features

Propose to amend Sections in Chapter 11B from the 2022 CBC into the 2025 CBC as listed below.

| DSA-AC 01/24 ITEM 13 | Code Section | CAC Action | Agency Response | Public Comments | Annotations | CBSC Action |
| --- | --- | --- | --- | --- | --- | --- |
| - 13-1 | 11B-809.6 Residential dwelling units with adaptable features. | Approve | Accept |  | Propose to amend section removing “multifamily” from the residential dwelling units with adaptable features. Terms defines the unit, not the building. |  |

#### ITEM 14, Chapter 11B, Section 11B-809.8 Doors

Propose to amend Sections in Chapter 11B from the 2022 CBC into the 2025 CBC as listed below.

| DSA-AC 01/24 ITEM 14 | Code Section | CAC Action | Agency Response | Public Comments | Annotations | CBSC Action |
| --- | --- | --- | --- | --- | --- | --- |
| - 14-1 | 11B-809.8 Doors. | Approve | Accept |  | Propose to amend this section to provide clarity for interior doors within residential dwelling units with adaptable features. |  |
| - 14-2 | 11B-809.8.1 Door thresholds. | Approve | Accept |  | Propose to amend this section to clarify doors intended for user passage shall have thresholds complying with Section 11B-404.2.5 |  |
| - 14-3 | 11B-809.8.2 Door opening force. | Approve | Accept |  | Propose to amend section and clarify doors to comply with Section 11B-404.2.9 and put existing language as an exception. |  |
| - 14-4 | 11B-809.8.3 Door maneuvering clearance. | Approve | Accept |  | Propose to amend this section to provide additional clarity to the code user maneuvering clearances shall extend the full width of the doorway. |  |
| - 14-5 | 11B-809.3.1 Floor or ground surface. | Approve | Accept |  | Adopt new section to clarify the requirements for maneuvering clearances at the floor or ground surface. |  |

#### ITEM 15, Chapter 11B, 11B-809.10 Bathrooms and powder rooms

Propose to amend Sections in Chapter 11B from the 2022 CBC into the 2025 CBC as listed below.

| DSA-AC 01/24 ITEM 15 | Code Section | CAC Action | Agency Response | Public Comments | Annotations | CBSC Action |
| --- | --- | --- | --- | --- | --- | --- |
| - 15-1 | 11B-809.10 Toilet and bathing rooms Bathroom and powder rooms. | Approve | Accept |  | Propose to repeal original title: Toilet and bathing rooms to Bathrooms and powder rooms. |  |
| - 15-2 | 11B-809.10.1 General | Approve | Accept |  | Propose to add additional reference to this section which requires backing for grab bars at bathtubs |  |
| - 15-3 | 11B-809.10.2 Number of complying bathing rooms bathrooms and fixtures. | Approve | Accept |  | Propose to change the reference to “toilet and bathing rooms” to “bathrooms” applicable to residential dwelling units with adaptable features. |  |
| - 15-4 | 11B-809.10.3 Powder rooms. | Approve | Accept |  | Propose to amend section removing references to grab bars in bathtubs and showers that don’t belong in a powder room. |  |
| - 15-5 | 11B-809.10.4 Sufficient maneuvering space. | Approve | Accept |  | Propose to amend section to repeal toilet and bathing rooms and add bathrooms. |  |
| - 15-6 | 11B-809.10.8 Lavatories, vanities, mirrors and towel bars. | Approve | Accept |  | Propose to amend section to repeal toilet and bathing rooms and add bathrooms. |  |

#### ITEM 16, Chapter 11B, section 11b-809.10.6.1 size

Propose to amend Sections in Chapter 11B from the 2022 CBC into the 2025 CBC as listed below.

| DSA-AC 01/24 ITEM 16 | Code Section | CAC Action | Agency Response | Public Comments | Annotations | CBSC Action |
| --- | --- | --- | --- | --- | --- | --- |
| - 16-1 | 11B-809.10.6.1 Size | Approve | Accept |  | Propose to amend section which incorrectly included requirements for fully accessible showers. Proposal brings for applicable requirements for showers. |  |

#### ITEM 17, Chapter 11B, 11B-809.10.6.3 Clearance

Propose to amend Sections in Chapter 11B from the 2022 CBC into the 2025 CBC as listed below.

| DSA-AC 01/24 ITEM 17 | Code Section | CAC Action | Agency Response | Public Comments | Annotations | CBSC Action |
| --- | --- | --- | --- | --- | --- | --- |
| - 17-1 | 11B-809.10.6.3 Clearance Floor space. | Approve as Amended | Accept |  | Propose to amend section to increase the clear floor space width from 30’ to 36” at showers in residential dwelling units with adaptable features. CAC: 9 Point Criteria #6. No suggested amendment for this section. After CAC: No amendments made to this section. |  |
| - 17-2 | FIGURE 11B-809.10.6.3 Transfer Type Shower Clearance. | Approve as Amended | Accept |  | Adopt new figure. CAC: 9 Point Criteria #6. Match the name of the figure with the name of the associated code section. After CAC: Changed the title of the figure from “Transfer Type Shower Clear Floor Space” to “Transfer Type Shower Clearance”. |  |

#### ITEM 18, Chapter 11B, section 11b-902.2.1 overlap

Propose to amend Sections in Chapter 11B from the 2022 CBC into the 2025 CBC as listed below.

| DSA-AC 01/24 ITEM 18 | Code Section | CAC Action | Agency Response | Public Comments | Annotations | CBSC Action |
| --- | --- | --- | --- | --- | --- | --- |
| - 18-1 | 11B-902.2.1 Overlap | Approve | Accept |  | Propose to adopt new section to prohibit overlap where the clear floor space and an accessible route are required in the dining area. |  |

#### ITEM 19, Chapter 15 rooftop structures, section 1511.9.6 accessibility and egress

Propose to amend Sections in Chapter 11B from the 2022 CBC into the 2025 CBC as listed below.

| DSA-AC 01/24 ITEM 19 | Code Section | CAC Action | Agency Response | Public Comments | Annotations | CBSC Action |
| --- | --- | --- | --- | --- | --- | --- |
| - 19-1 | 1511.9.6 Accessibility and egress. | Approve | Accept |  | New code section to be amended to repeal the reference to Chapter 11 and provide a reference California adopted Chapters 11A and 11B. |  |

### 2025 CALIFORNIA EXISTING BUILDING CODE
ITEMS 20 through 21

#### ITEM 20, Chapter 3 PROVISIONS FOR ALL COMPLIANCE METHODS, Section 306 Accessibility for existing buildings

Propose not to adopt Section 306 from the 2024 IEBC and carry forward existing amendments with modifications listed below.

| DSA-AC 01/24 ITEM 20 | Code Section | CAC Action | Agency Response | Public Comments | Annotations | CBSC Action |
| --- | --- | --- | --- | --- | --- | --- |
| - 20-1 | 306 Accessibility for existing buildings. | Approve | Accept |  | Propose to remove the text of IEBC Section 306 et seq. and amend existing DSA-AC language in this section to state that public buildings, public accommodations, commercial facilities, and public housing are required to comply with Chapter 11B. |  |

#### ITEM 21, Chapter 16 referenced standards

Propose to amend Chapter 16 from the 2024 IEBC as listed below.

| DSA-AC 01/24 ITEM 21 | Code Section | CAC Action | Agency Response | Public Comments | Annotations | CBSC Action |
| --- | --- | --- | --- | --- | --- | --- |
| - 21-1 | ICC A117.1-17 Accessible and Usable Buildings and Facilities | Approve | Accept |  | DSA does not adopt ICCs referenced standard A117.1-17 for accessibility requirements. DSA amends the referenced standard eliminating the Section 306 subsections that reference ICC A117.1. |  |