# COMMISSION ACTION MATRIX 
BUILDING, FIRE AND OTHER – STRUCTURAL SAFETY/LATERAL FORCES aD-hoc (bfo/SDlF) CODE ADVISORY COMMITTEE – fEBRUARY 17-18, 2026

## 2025 California FIRE CODE, TITLE 24, PART 9
STATE FIRE MARSHAL, SFM 03/25

### LEGEND:

CAC Actions: Approve, Disapprove, Approve as Amended, Further Study Required

Agency Responses: Accept, Disagree, Withdraw

CBSC Actions: Approve, Disapprove, Approve as Amended, Further Study Required

Matrix Paper Color (for commission action only): GREEN = uncontested items, YELLOW = challenged items, SALMON = withdrawn, no action required

If using assistive technology, please adjust your settings to recognize underline, strikeout, italic and ellipsis.

- Model Code language appears upright

- Existing California amendments appear in italic

- Amended model code or new California amendments appear underlined & italic

- Repealed model code language appears upright and in strikeout

- Repealed California amendments appear in italic and strikeout

- Ellipses (…) indicate existing text remains unchanged

### ITEM 1
Chapter 2 – DEFINITIONS

SFM proposes to amend definitions listed below.

| SFM 03/25 ITEM 1 | Code Section | CAC Action | Agency Response | Public Comments | Annotations | CBSC Action |
| --- | --- | --- | --- | --- | --- | --- |
| 1-1 | CARBON MONOXIDE SOURCE Carbon Monoxide Source, Direct Carbon Monoxide Source, Forced-indirect |  |  |  | Amend the model code definition to clarify and differentiate between direct and indirect CO sources. |  |
| 1-2 | CHILD-CARE. CHILD CARE CENTER. DAY CHILD-CARE HOME, FAMILY. DAY CHILD-CARE HOME, LARGE FAMILY. DAY CHILD-CARE HOME, SMALL FAMILY. DAY CARE. |  |  |  | Amend to align with other existing regulations and laws. |  |
| 1-3 | COMBUSTIBLE LIQUID. FLAMMABLE LIQUID. |  |  |  | Amend model definitions to align with the federal Occupational Safety and Health Administration (OSHA) hazardous materials regulations and Globally Harmonized System of Classification and Labelling of Chemicals (GHS). |  |
| 1-4 | COMPRESSED GAS. COMPRESSED GAS, DISSOLVED. COMPRESSED GAS, GASEOUS. COMPRESSED GAS, LIQUEFIED. |  |  |  | Amend model definitions to align with the federal Occupational Safety and Health Administration (OSHA) hazardous materials regulations and Globally Harmonized System of Classification and Labelling of Chemicals (GHS). |  |
| 1-5 | CORROSIVE. |  |  |  | Amend model definition to align with the federal Occupational Safety and Health Administration (OSHA) hazardous materials regulations and Globally Harmonized System of Classification and Labelling of Chemicals (GHS). |  |
| 1-6 | CRYOGENIC FLUID. CRYOGENIC FLUID, FLAMMABLE. CRYOGENIC FLUID, INERT. CRYOGENIC FLUID, OXIDIZING. FLAMMABLE CRYOGENIC FLUID. OXIDIZING CRYOGENIC FLUID. |  |  |  | Amend model definitions to align with the federal Occupational Safety and Health Administration (OSHA) hazardous materials regulations and Globally Harmonized System of Classification and Labelling of Chemicals (GHS). |  |
| 1-7 | EXPLOSIVE. |  |  |  | Amend model definition to align with the federal Occupational Safety and Health Administration (OSHA) hazardous materials regulations and Globally Harmonized System of Classification and Labelling of Chemicals (GHS). |  |
| 1-8 | FLAMMABLE GAS. |  |  |  | Amend model definition to align with the federal Occupational Safety and Health Administration (OSHA) hazardous materials regulations and Globally Harmonized System of Classification and Labelling of Chemicals (GHS). |  |
| 1-9 | FLAMMABLE SOLID. |  |  |  | Amend model definition to align with the federal Occupational Safety and Health Administration (OSHA) hazardous materials regulations and Globally Harmonized System of Classification and Labelling of Chemicals (GHS). |  |
| 1-10 | HIGHLY TOXIC. TOXIC. |  |  |  | Amend model definitions to align with the federal Occupational Safety and Health Administration (OSHA) hazardous materials regulations and Globally Harmonized System of Classification and Labelling of Chemicals (GHS). |  |
| 1-11 | INFANT. |  |  |  | Amend to align with Title 22. |  |
| 1-12 | NON-PATIENT-CARE SUITE. |  |  |  | Editorial modification to avoid misunderstanding. |  |
| 1-13 | ORGANIC PEROXIDE. |  |  |  | Amend model definition to align with the federal Occupational Safety and Health Administration (OSHA) hazardous materials regulations and Globally Harmonized System of Classification and Labelling of Chemicals (GHS). |  |
| 1-14 | OXIDIZER. |  |  |  | Amend model definition to align with the federal Occupational Safety and Health Administration (OSHA) hazardous materials regulations and Globally Harmonized System of Classification and Labelling of Chemicals (GHS). |  |
| 1-15 | OXIDIZING GAS |  |  |  | Amend model definition to align with the federal Occupational Safety and Health Administration (OSHA) hazardous materials regulations and Globally Harmonized System of Classification and Labelling of Chemicals (GHS). |  |
| 1-16 | PYROPHORIC. |  |  |  | Amend model definition to align with the federal Occupational Safety and Health Administration (OSHA) hazardous materials regulations and Globally Harmonized System of Classification and Labelling of Chemicals (GHS). |  |
| 1-17 | UNSTABLE (REACTIVE) MATERIAL. |  |  |  | Amend model definition to align with the federal Occupational Safety and Health Administration (OSHA) hazardous materials regulations and Globally Harmonized System of Classification and Labelling of Chemicals (GHS). |  |
| 1-18 | WATER-REACTIVE MATERIAL. |  |  |  | Amend model definition to align with the federal Occupational Safety and Health Administration (OSHA) hazardous materials regulations and Globally Harmonized System of Classification and Labelling of Chemicals (GHS). |  |

### ITEM 2
Chapter 2 – DEFINITIONS, SECTION 203—OCCUPANCY CLASSIFICATION AND USE

SFM proposes to amend sections listed below.

| SFM 03/25 ITEM 2 | Code Section | CAC Action | Agency Response | Public Comments | Annotations | CBSC Action |
| --- | --- | --- | --- | --- | --- | --- |
| 2-1 | 203.4.2 Group E, child-care facilities. 203.4.2.2 Six Five or fewer children. 204.4.2.3 Six Five or fewer children in a dwelling unit. |  |  |  | Amend to align with Title 22 and to align references to “six or fewer” persons in Part 2 and Part 9. |  |
| 2-2 | 203.7.4 Institutional Group I-4, day care and child care facilities. 203.7.4.1 Within places of worship. 203.7.4.2 Six Five or fewer persons receiving care. 203.7.4.3 Six Five or fewer persons receiving care in a dwelling unit. |  |  |  | Amend to include child care facilities, align with existing statutes, align references to “six or fewer” persons in Part 2 and Part 9 and clarify the application of the code when the number of care recipients is limited, thereby reducing hazard levels. |  |

### ITEM 3
Chapter 9 – FIRE PROTECTION AND LIFE SAFETY SYSTEMS, SECTION 907 FIRE ALARM AND DETECTION SYSTEMS

SFM proposes to amend sections listed below.

| SFM 03/25 ITEM 3 | Code Section | CAC Action | Agency Response | Public Comments | Annotations | CBSC Action |
| --- | --- | --- | --- | --- | --- | --- |
| 3-1 | 907.2.5.1 Group H occupancies located on the 11th story and above. 907.2.6.3.4 System annunciation. 907.2.28.1 Group L occupancies located on the 11th story and above. 907.6.4 Zones. |  |  |  | Repeal or relocate outdated or duplicative amendments as these requirements already addressed in adopted referenced standards. Amend to clarify and align with national standards. |  |

### ITEM 4
Chapter 9 – FIRE PROTECTION AND LIFE SAFETY SYSTEMS, SECTION 915 – CARBON MONOXIDE (CO) DETECTION

SFM proposes to amend sections listed below.

| SFM 03/25 ITEM 4 | Code Section | CAC Action | Agency Response | Public Comments | Annotations | CBSC Action |
| --- | --- | --- | --- | --- | --- | --- |
| 4-1 | SECTION 915 – CARBON MONOXIDE (CO) DETECTION with all the subsections |  |  |  | Amend Section 915 replacing model code language and existing the majority of SFM amendments with new language. |  |

### ITEM 5
Chapter 10 – MEANS OF EGRESS, SECTION 1010 – DOORS, GATES AND TURNSTILES

SFM proposes to amend sections listed below.

| SFM 03/25 ITEM 5 | Code Section | CAC Action | Agency Response | Public Comments | Annotations | CBSC Action |
| --- | --- | --- | --- | --- | --- | --- |
| 5-1 | 1010.2.12 Delayed egress. |  |  |  | Delete model language “or heat” in the first paragraph. |  |

### ITEM 6
Chapter 11 – CONSTRUCTION REQUIREMENTS FOR EXISTING BUILDINGS, SECTION 1103 – FIRE SAFETY REQUIREMENTS FOR EXISTING BUILDINGS

SFM proposes to amend sections listed below.

| SFM 03/25 ITEM 6 | Code Section | CAC Action | Agency Response | Public Comments | Annotations | CBSC Action |
| --- | --- | --- | --- | --- | --- | --- |
| 6-1 | ### 1103.9 Carbon monoxide detection. |  |  |  | Amend to coordinate with Part 2 and Part 9, Section 915 Carbon Monoxide language. |  |

### ITEM 7
Chapter 12 – ENERGY SYSTEMS, SECTION 1207 – ELECTRICAL ENERGY STORAGE SYSTEMS (ESS)

SFM proposes to amend sections listed below.

| SFM 03/25 ITEM 7 | Code Section | CAC Action | Agency Response | Public Comments | Annotations | CBSC Action |
| --- | --- | --- | --- | --- | --- | --- |
| 7-1 | ### Multiple sections (see ET) |  |  |  | Amend to delete all pointers referencing to 2023 Edition of NFPA 855 to coordinate with the editions of standards referenced in Chapter 80. |  |

### ITEM 8
Chapter 50 – HAZARDOUS MATERIALS-GENERAL PROVISIONS

SFM proposes to amend sections listed below.

| SFM 03/25 ITEM 8 | Code Section | CAC Action | Agency Response | Public Comments | Annotations | CBSC Action |
| --- | --- | --- | --- | --- | --- | --- |
| 8-2 | 5001.1 Scope. |  |  |  | Amend to add a phrase “or storage occupancies” to Exception #2 for alcoholic beverages in retail or wholesale sales occupancies. |  |
| 8-2 | 5003.1.1 Maximum allowable quantity per control area. |  |  |  | Amend the Exception regarding medical gas cylinder sizes aligning with language used in NFPA 99 in collaboration with OSHPD. |  |

### ITEM 9
Chapter 80 – REFERENCED STANDARDS

SFM proposes to amend Referenced Standards listed below.

| SFM 03/25 ITEM 9 | Code Section | CAC Action | Agency Response | Public Comments | Annotations | CBSC Action |
| --- | --- | --- | --- | --- | --- | --- |
| 9-1 | NFPA 72-25 National Fire Alarm Signaling Code, Section 12.4.2 Pathway Survivability 1. |  |  |  | Add amendment to clarify requirements for “Pathway Survivability Level 1.” |  |
| 9-2 | NFPA 72-25 National Fire Alarm Signaling Code, Section 26.2.11.3 |  |  |  | Add amendment to delete NFPA 72-25 Section 26.2.11.3. |  |
| 9-3 | NFPA 855 – 23 26 Standard for the Installation of Stationary Energy Storage Systems |  |  |  | Adopt the most recent edition of NFPA 855. |  |
| 9-4 | ASHRAE 15 – 2022 2024; including Addendum A: Safety Standard for Refrigeration Systems |  |  |  | Adopt the most recent edition of ASHRAE 15. |  |