# CALIFORNIA BUILDING STANDARDS COMMISSION
September 1, 2026
CALGREEN EV WORKSHOP #1

Agenda Item 3c
CALIFORNIA GREEN BUILDING STANDARDS CODE,
(CALGreen), PART 11,
CALIFORNIA BUILDING STANDARDS CODE,
TITLE 24, CALIFORNIA CODE OF REGULATIONS

## AGENDA ITEM 3c

The state agency shall draft the regulations in plain, straightforward language, avoiding technical terms as much as possible and using a coherent and easily readable style. The agency shall draft the regulation in plain English. A notation shall follow the express terms of each regulation listing the specific statutes authorizing the adoption and listing specific statutes being implemented, interpreted, or made specific (Government Code Section 11346.2(a)(1)).

If using assistive technology, please adjust your settings to recognize underline, strikeout and ellipsis.

## LEGEND for EXPRESS TERMS

- Existing California language appears upright.

- Proposed amendments appear in underline and strikeout.

- Ellipses ( ...) indicate existing text remains unchanged.

- Rationale: The justification for the change is shown after each section or series of related changes.

### ITEM 3c
Chapter 5, NONRESIDENTIAL MANDATORY MEASURES,
DIVISION 5.5 – ENVIRONMENTAL QUALITY,
SECTION 5.504 – POLLUTANT CONTROL

5.504.4 Finish material pollutant control. Finish materials shall comply with Sections 5.504.4.1 through 5.504.4.6.

5.504.4.1 Adhesives, sealants and caulks. Adhesives, sealants and caulks used on the project shall meet the requirements of the following standards:

1. Adhesives, adhesive bonding primers, adhesive primers, sealants, sealant primers and caulks shall comply with local or regional air pollution control or air quality management district rules where applicable, or SCAQMD Rule 1168 VOC limits, as shown in Tables 5.504.4.1 and 5.504.4.2. Such products also shall comply with the Rule 1168 prohibition on the use of certain toxic compounds (chloroform, ethylene dichloride, methylene chloride, perchloroethylene and trichloroethylene), except for aerosol products as specified in subsection standard 2, below.

2. Aerosol adhesives, and smaller unit sizes of adhesives, and sealant or caulking compounds (in units of product, less packaging, which do not weigh more than one pound and do not consist of more than 16 fluid ounces) shall comply with statewide VOC standards and other requirements, including prohibitions on use of certain toxic compounds, of California Code of Regulations, Title 17, commencing with Section 94507.

| TABLE 5.504.4.1—ADHESIVE VOC LIMIT1,2—Less Water and Less Exempt Compounds in Grams Per Liter |  |
| --- | --- |
| ARCHITECTURAL APPLICATIONS | CURRENT VOC LIMIT |
| Indoor carpet adhesives | 50 |
| Carpet pad adhesives | 50 |
| Outdoor carpet adhesives | 150 |
| Wood flooring adhesive | 100 20 |
| Rubber floor adhesives | 60 |
| Subfloor adhesives | 50 |
| Ceramic tile adhesives | 65 |
| VCT and asphalt tile adhesives | 50 |
| Drywall and panel adhesives | 50 |
| Cove base adhesives | 50 |
| Multipurpose construction adhesives | 70 |
| Structural glazing adhesives | 100 |
| Single-ply roof membrane adhesives | 250 |
| Other adhesive not specifically listed | 50 |
| SPECIALTY APPLICATIONS |  |
| PVC welding | 510 425 |
| CPVC welding | 490 400 |
| ABS welding | 325 |
| Plastic cement welding | 250 |
| Adhesive primer for plastic | 550 |
| Contact adhesive | 80 |
| Special purpose contact adhesive | 250 |
| Structural wood member adhesive | 140 |
| Top and trim adhesive | 250 |
| SUBSTRATE SPECIFIC APPLICATIONS |  |
| Metal to metal | 30 |
| Plastic foams | 50 |
| Porous material (except wood) | 50 |
| Wood | 30 |
| Fiberglass | 80 |
| 1. If an adhesive is used to bond dissimilar substrates together the adhesive with the highest VOC content shall be allowed. 2. Values established in this table are based on SCAQMD Rule 1168 (Amended November 4, 2022). Any subsequent amendments to the referenced document do not apply to this table. http://www.arb.ca.gov/DRDB/SC/CURHTML/R1168.PDF. |  |

| TABLE 5.504.4.2—SEALANT VOC LIMIT1—Less Water and Less Exempt Compounds in Grams per Liter |  |
| --- | --- |
| SEALANTS | CURRENT VOC LIMIT |
| Architectural | 250 |
| Marine deck | 760 |
| Nonmembrane roof | 300 |
| Roadway | 250 |
| Single-ply roof membrane | 450 |
| Other | 420 |
| SEALANT PRIMERS |  |
| Architectural Nonporous Porous | 250 775 |
| Modified bituminous | 500 |
| Marine deck | 760 |
| Other | 750 |
| 1. Values established in this table are based on SCAQMD Rule 1168 (Amended November 4, 2022). Any subsequent amendments to the referenced document do not apply to this table. Note: For additional information regarding methods to measure the VOC content specified in these tables, see South Coast Air Quality Management District Rule 1168. |  |

5.504.4.3 Paints and coatings. Architectural paints and coatings shall comply with VOC limits in Table 1 of the ARB Architectural Coatings Suggested Control Measure, as shown in Table 5.504.4.3, unless more stringent local limits apply. The VOC content limit for coatings that do not meet the definitions for the specialty coatings categories listed in Table 5.504.4.3 shall be determined by classifying the coating as a Flat, and Nonflat or Nonflat-High Gloss coating, based on its gloss, as defined in Subsections 4.21, 4.36 and 4.37  4.23 and 4.39 of the 2007 May 2020 California Air Resources Board Suggested Control Measure, and the corresponding Flat, and Nonflat or Nonflat-High Gloss VOC limit in Table 5.504.4.3 shall apply.

| TABLE 5.504.4.3—VOC CONTENT LIMITS FOR ARCHITECTURAL COATINGS2, 3 Grams of VOC per Liter of Coating, Less Water and Less Exempt Compounds |  |
| --- | --- |
| COATING CATEGORY | CURRENT LIMIT |
| Flat coatings | 50 |
| Nonflat coatings | 100 50 |
| Nonflat-high gloss coatings | 150 |
| SPECIALTY COATINGS |  |
| Aluminum roof coatings | 400 100 |
| Basement specialty coatings | 400 |
| Bituminous roof coatings | 50 |
| Bituminous roof primers | 350 |
| Bond breakers | 350 |
| Concrete curing compounds | 350 |
| Concrete/masonry sealers | 100 |
| Driveway sealers | 50 |
| Dry fog coatings | 150 50 |
| Faux finishing coatings | 350 |
| Fire resistive coatings | 350 150 |
| Floor coatings | 100 50 |
| Form-release compounds | 250 100 |
| Graphic arts coatings (sign paints) | 500 |
| High temperature coatings | 420 |
| Industrial maintenance coatings | 250 |
| Low solids coatings1 | 120 |
| Magnesite cement coatings | 450 |
| Mastic texture coatings | 100 |
| Metallic pigmented coatings | 500 |

| TABLE 5.504.4.3—VOC CONTENT LIMITS FOR ARCHITECTURAL COATINGS2, 3 Grams of VOC per Liter of Coating, Less Water and Less Exempt Compounds—continued |  |
| --- | --- |
| SPECIALTY COATINGS |  |
| Multicolor coatings | 250 |
| Pretreatment wash primers | 420 |
| Primers, sealers and undercoaters | 100 |
| Reactive penetrating sealers | 350 |
| Recycled coatings | 250 |
| Roof coatings | 50 |
| Rust preventative coatings | 250 |
| Shellacs Clear Opaque | 730 550 |
| Specialty primers, sealers and undercoaters | 100 |
| Stains Exterior Interior | 250 100 250 |
| Stone consolidants | 450 |
| Swimming pool coatings | 340 |
| Traffic marking coatings | 100 |
| Tub and tile refinish coatings | 420 |
| Waterproofing membranes | 250 100 |
| Wood coatings | 275 |
| Wood preservatives | 350 |
| Zinc-rich primers | 340 |
| 1. Grams of VOC per liter of coating, including water and including exempt compounds. 2. The specified limits remain in effect unless revised limits are listed in subsequent columns in the table. 3. Values in this table are derived from those specified by the California Air Resources Board, Architectural Coatings Suggested Control Measure, February 1, 2008 May 2020 . More information is available from the Air Resources Board. |  |

5.504.4.3.1 Aerosol paints and coatings. Aerosol paints and coatings shall meet the Product-Weighted Maximum Incremental Reactivity  (PWMIR) Limits for ROC Reactivity limits (RL) in Section 94522(a)(3) and other requirements, including prohibitions on use of certain toxic compounds and ozone depleting substances, in Sections 94522(c)(2)  94522(d)(2) and (d)(2) (e)(2) of California Code of Regulations, Title 17, commencing with Section 94520.; and in areas Areas under the jurisdiction of the Bay Area Air Quality Management District shall additionally comply with the percent VOC by weight of product limits of Regulation 8 Rule 49.

5.504.4.3.2 Verification. Verification of compliance with this section shall be provided at the request of the enforcing agency. Documentation may include, but is not limited to, the following:

1. Manufacturer’s product specification

2. Field verification of on-site product containers

5.504.4.4 Carpet systems. All carpet…Specification 01350).

### Rationale:

BSC is proposing to amend various tables for Adhesives, sealants and caulks, VOC content limits for architectural coatings and formaldehyde limits.

The proposed changes are needed to update the existing values in the 2025 CALGreen code to align with the latest product values from the California Consumer Products Regulation (California Code of Regulations, Title 17, commencing with Section 94507), Air Quality Management District, Rule 1168. Adhesive and Sealant Applications, and California Air Resources Board (CARB) Suggested Control Measure for Architectural Coatings.

BSC staff coordinated with staff from CARB, SQMD and DSA staff.

These proposed changes are needed to ensure that the CALGreen code contains the most current finish material control product values.

Updating the tables provides code users with accurate product information that can be specified to ensure code compliance.

The proposed changes are non-substantive and have no regulatory effect because the proposed changes are simply updating the VOC limits to align with the current law.