# CALIFORNIA BUILDING STANDARDS COMMISSION
October 29, 2026
CALGREEN WORKSHOP #2

Agenda Item 3f
CALIFORNIA GREEN BUILDING STANDARDS CODE,
(CALGreen), PART 11,
CALIFORNIA BUILDING STANDARDS CODE,
TITLE 24, CALIFORNIA CODE OF REGULATIONS

The state agency shall draft the regulations in plain, straightforward language, avoiding technical terms as much as possible and using a coherent and easily readable style. The agency shall draft the regulation in plain English. A notation shall follow the express terms of each regulation listing the specific statutes authorizing the adoption and listing specific statutes being implemented, interpreted, or made specific (Government Code Section 11346.2(a)(1)).

If using assistive technology, please adjust your settings to recognize underline, strikeout and ellipsis.

## LEGEND for EXPRESS TERMS

- Existing California language appears upright.

- Proposed amendments appear in underline and strikeout.

- Ellipses ( ...) indicate existing text remains unchanged.

- Rationale: The justification for the change is shown after each section or series of related changes.

## DRAFT EXPRESS TERMS

### AGENDA ITEM 3f
Clarification of CALGreen nonresidential tier scoping and adoption

CHAPTER 1 ADMINISTRATION

…

101.4 Appendices. Provisions contained in the appendices of this code are not mandatory unless specifically adopted by a city, county, or city and county in compliance with Health and Safety Code Sections 18930 and 18941.5, respectively, for Building Standards Law; Health and Safety Code Section 17950 for State Housing Law; and Health and Safety Code Section 13869.7 for Fire Protection Districts. See Section 101.7 of this code.

…

101.7 City, county, or city and county amendments, additions or deletions. This code is intended to set mandatory minimum Green Building Standards and includes optional tiers that may, at the discretion of any city, county, or city and county, be applied. Local jurisdictions adopting optional tier provisions as mandatory standards, with or without additional amendments, shall make express findings compliant with Section 101.7.1. This code does not limit the authority of city, county, or city and county governments to make necessary changes to the provisions contained in this code pursuant to Section 101.7.1. The effective date of amendments, additions or deletions to this code for cities, counties, or cities and counties filed pursuant to Section 101.7.1 shall be the date on which it is filed. However, in no case shall the amendments, additions or deletions to this code be effective any sooner than the effective date of this code.

Local modifications shall comply with Health and Safety Code Section 18941.5(b) for Building Standards Law, Health and Safety Code Section 17958.5 for State Housing Law or Health and Safety Code Section 13869.7 for Fire Protection Districts.

101.7.1 Findings and filings.

1.	The city, county, or city and county shall make express findings for adoption of provisions contained in the optional tiers as mandatory standards and for each amendment, addition or deletion based upon climatic, topographical or geological conditions. For the purpose of this section, climatic, topographical or geological conditions include local environmental conditions as established by the city, county, or city and county.

2.	The city, county, or city and county shall file the adoption of provisions contained in the optional tiers, amendments, additions or deletions expressly marked and identified as to the applicable findings. Cities, counties, cities and counties, and fire departments shall file the amendments, additions or deletions and the findings with the California Building Standards Commission at 2525 Natomas Park Drive, Suite 130, Sacramento, CA 95833.

…

APPENDIX A5 NONRESIDENTIAL VOLUNTARY MEASURES

DIVISION A5.1 – PLANNING AND DESIGN

PREFACE Given that land use and planning …

SECTION A5.101 – GENERAL

A5.101.1Scope. The provisions of this chapter outline planning, design and development methods that include environmentally responsible site selection, building design, building siting and development to protect, restore and enhance the environmental quality of the site and respect the integrity of adjacent properties. Division A5.1 provides voluntary measures for planning, design and development. To achieve CALGreen Tier 1 or Tier 2, a project must comply with all mandatory measures in Chapter 5 and certain prerequisite and elective measures in this division. The required prerequisite and elective measures are specified in Section A5.601.

…

DIVISION A5.2 – ENERGY EFFICIENCY

SECTION A5.201 – GENERAL

A5.201.1 Scope. For the purposes of mandatory energy efficiency standards in this code, the California Energy Commission will continue to adopt mandatory standards. It is the intent… …is available at https://www.energy.ca.gov/programs-and-topics/programs/building-energy-efficiency-standards/2025-building-energy-efficiency

Division A5.2 provides voluntary measures for energy efficiency. To achieve CALGreen Tier 1 or Tier 2, a project must comply with all mandatory measures in Chapter 5 and certain prerequisite and elective measures in this division. The required prerequisite and elective measures are specified in Section A5.601.

…

DIVISION A5.3 – WATER EFFICIENCY AND CONSERVATION

SECTION A5.301 – GENERAL

A5.301.1 Scope. Division A5.3 provides voluntary measures for conserving water used indoors, outdoors and in wastewater conveyance. To achieve CALGreen Tier 1 or Tier 2, a project must comply with all mandatory measures in Chapter 5 and certain prerequisite and elective measures in this division. The required prerequisite and elective measures are specified in Section A5.601.

…

DIVISION A5.4 – MATERIAL CONSERVATION AND RESOURCE EFFICIENCY

SECTION A5.401 – GENERAL

A5.401.1 Scope. The provisions of this chapter specify the requirements of achieving enhanced compliance with material conservation, resource efficiency, and greenhouse gas (GHG) emissions reduction through reuse of existing building stock and materials; use of recycled, regional, rapidly renewable and certified wood materials; and employment of techniques to reduce pollution through recycling of materials. Division A5.4 provides voluntary measures for material conservation, resource efficiency, and greenhouse gas (GHG) emissions reduction. To achieve CALGreen Tier 1 or Tier 2, a project must comply with all mandatory measures in Chapter 5 and certain prerequisite and elective measures in this division. The required prerequisite and elective measures are specified in Section A5.601.

…

DIVISION A5.5 – ENVIRONMENTAL QUALITY

SECTION A5.501 – GENERAL

A5.501.1Scope. The provisions of this chapter shall outline means of reducing the quantity of air contaminants that are odorous, irritating and/or harmful to the comfort and well-being of a building’s installers, occupants and neighbors. Division A5.5 provides voluntary measures for improving indoor and outdoor air quality and environmental comfort. To achieve CALGreen Tier 1 or Tier 2, a project must comply with all mandatory measures in Chapter 5 and certain prerequisite and elective measures in this division. The required prerequisite and elective measures are specified in Section A5.601.

…

DIVISION A5.6 – VOLUNTARY TIERS

SECTION A5.601 – CALGreen TIER 1 AND TIER 2

A5.601.1 Scope. The measures contained in this appendix are not mandatory unless adopted by local government as specified in Section 101.7. Local jurisdictions adopting optional tier provisions as mandatory standards, with or without additional amendments, shall make express findings compliant with Section 101.7.1.

The provisions of this section outline means of achieving enhanced construction or reach levels by incorporating additional green building measures for newly constructed nonresidential buildings as well as additions and alterations. In order to meet one of the tier levels designers, builders or property owners are required to incorporate additional green building measures necessary to meet the threshold of each level. Refer to the provisions in Section 301.3 for nonresidential additions and alterations scope and application.

A5.601.2 CALGreen Tier 1

A5.601.2.1 Prerequisites and electives. To achieve CALGreen Tier 1 tier status, a project must meet all of the mandatory measures in Chapter 5 and the prerequisite and elective measures from Divisions A5.1 through A5.5 as required by Section A5.601.2.2., in addition, meet the provisions of this section.

A5.601.2.2 Energy performance. For the purposes of mandatory energy efficiency standards in this code, the California Energy Commission will continue to adopt mandatory standards. [This language is duplicative of the scoping language in Section A5.201.1, and does not need to be duplicated here.]

A5.601.2.3 Tier 1. Comply with the energy efficiency requirements in Section A5.203.1.1 and Section A5.203.1.2.1. [This language is redundant with the requirements listed below for Division A5.2. All scoping for Tier 1 should be included in the list in A5.601.2.2, and not in this section]

A5.601.2.2 A5.601.2.4 Voluntary measures for Tier 1. In addition to the provisions of Sections A5.601.2.1 and A5.601.2.3 above, c Compliance with the following voluntary measures from Appendix A5 is required for Tier 1:

- From Division A5.1,

  - Comply with the designated parking requirements for high-efficiency vehicles for a minimum of 35 percent of parking capacity per Section A5.106.5.1.

  - Electric vehicle (EV) charging [N] and Table A5.106.5.3.1 w/footnotes.

  - Comply with thermal emittance, solar reflectance or SRI values for cool roofs in Section A5.106.11.2 and Table A5.106.11.2.2 A5.106.11.2(1).1

  - Comply with one elective measure selected from this division.

- From Division A5.2 comply with Section A5.203.1.2.1, if applicable, and ONE of the following:

- 1. Outdoor lighting as described in A5.203.1.1.1.

- 2. Service water heating in restaurants as described in A5.203.1.1.2.

- 3. Warehouse Dock Seal Doors as described in A5.203.1.1.3.

- 4. Daylight Design Power Adjustments as described in 5.203.1.1.4.

- 5. Exhaust Air Heat Recovery as described in A5.203.1.1.5.

[Change the list numbering to letters for this item to match the overall organization of the list in this section.]

- From Division A5.3,

  - [See SB 745 proposal.]

  - [See SB 745 proposal.]

- From Division A5.4,

  - Comply with recycled content of 10 percent of materials based on estimated total cost, or use two products from Table A5.405.4 for at least 75 percent by cost in Section A5.405.4.

  - Comply with the 65-percent reduction in construction and demolition waste in Section A5.408.3.1.

  - Comply with one elective measure selected from this division.

- From Division A5.5,

  - Comply with resilient flooring systems for 90 percent of resilient flooring in Section A5.504.4.7.

  - Comply with thermal insulation meeting 2009 CHPS low-emitting materials list in Section A5.504.4.8.

  - Comply with one elective measure selected from this division.

- Comply with one additional elective measure selected from any division.

1 Cool roof is required for compliance with Tiers 1 and 2 and may be used to meet energy standards in Part 6, exceed energy standards and to mitigate heat island effect.

A5.601.3 CALGreen Tier 2.

A5.601.3.1 Prerequisites and electives. To achieve CALGreen Tier 2 tier status, a project must meet all of the mandatory measures in Chapter 5 and the prerequisite and elective measures from Divisions A5.1 through A5.5 as required by Section A5.601.3.2., in addition, meet the provisions of this section.

A5.601.3.2 Energy performance. For the purposes of mandatory energy efficiency standards in this code, the California Energy Commission will continue to adopt mandatory standards.[This language is duplicative of the scoping language in Section A5.201.1, and does not need to be duplicated here.]

A5.601.3.3 Tier 2. Comply with the energy efficiency requirements in Section A5.203.1.1 and Section A5.203.1.2.2.[This language is redundant with the requirements listed below for Division A5.2. All scoping for Tier 2 should be included in the list in A5.601.3.2, and not in this section]

A5.601.3.2 A5.601.3.4 Voluntary measures for Tier 2. In addition to the provisions of Sections A5.601.3.1 and A5.601.3.3 above, c Compliance with the following voluntary measures from Appendix A5 is required for Tier 2:

- From Division A5.1,

  - Comply with the designated parking requirements for fuel efficient vehicles for a minimum of 50 percent of parking capacity per Section A5.106.5.1.

  - Electric vehicle (EV) charging [N] and Table A5.106.5.3.2 with footnotes.

  - Comply with thermal emittance, solar reflectance or SRI values for cool roofs in Section A5.106.11.2 and Table A5.106.11.2.3 A5.106.11.2(2).1

  - Comply with three elective measures selected from this division.

- From Division A5.2 comply with Section A5.203.1.2.2, if applicable, and TWO of the following:

- 1. Outdoor lighting as described in A5.203.1.1.1.

- 2. Service water heating in restaurants as described in A5.203.1.1.2.

- 3. Warehouse Dock Seal Doors as described in A5.203.1.1.3.

- 4. Daylight Design Power Adjustments as described in 5.203.1.1.4.

- 5. Exhaust Air Heat Recovery as described in A5.203.1.1.5.

[Change the list numbering to letters for this item to match the overall organization of the list in this section.]

- From Division A5.3,

  - [See SB 745 proposal.]

  - [See SB 745 proposal.]

- From Division A5.4,

  - Comply with recycled content of 15 percent of materials based on estimated total cost, or use two products from Table A5.405.4 for at least 75 percent by cost in Section A5.405.4.1.

  - Comply with the 80-percent reduction in construction and demolition waste in Section A5.408.3.1.

  - Comply with three elective measures selected from this division.

- From Division A5.5,

  - Comply with resilient flooring systems for 100 percent of resilient flooring in Section A5.504.4.7.1.

Exception: Allowance may be permitted in Tier 2 for up to 5-percent specialty purpose flooring.

  - Comply with thermal insulation meeting 2009 CHPS low-emitting materials list and no added formaldehyde in Section A5.504.4.8.1.

  - Comply with three elective measures selected from this division.

- Comply with three additional elective measures selected from any division.

1 Cool roof is required for compliance with Tiers 1 and 2 and may be used to meet energy standards in Part 6, exceed energy standards and to mitigate heat island effect.

A5.601.4 Compliance verification. Compliance verification for with Section A5.601.2 or A5.601.3 shall be as required in Chapter 7 of this code. Compliance documentation shall be made part of the project record as required in Section 5.410.2 or 5.410.3 5.410.4.

TABLE A5.601 NONRESIDENTIAL BUILDINGS: Green Building Standards Code Proposed Performance Approach 
[Relocated to after the end of Section A5.601, as opposed to before it. This is consistent with the location of tables after the associated code language throughout Title 24.]

…

A5.602 – CALGreen VERIFICATION GUIDELINES MANDATORY MEASURES CHECKLIST

Application: This checklist shall be used for nonresidential projects that meet one of the following: new construction, building additions of 1,000 square feet or greater, or building alterations with…

…

#### RATIONALE:

BSC is proposing amendments in Chapter 1 to clarify that local jurisdictions adopting optional tier provisions from voluntary CALGreen appendices as mandatory standards in their local ordinances must make express findings based upon climatic, topographical, geological or environmental conditions even if no additional amendment, addition or deletion are made to the voluntary appendix language adopted by state agencies.

BSC is proposing amendments in Appendix A5 to clarify the scoping requirements for the voluntary CALGreen Tiers by providing a pointer to the Tier 1 and Tier 2 requirements in Section A5.601 from the start of each Division in Appendix A5 which contains voluntary measures (Division A5.1 through Division A5.5). This change is intended to have no material effect on the Tier requirements, while more clearly presenting the scoping information.