DIVISION OF THE STATE ARCHITECT

2018	TRIENNIAL CODE CYCLE

GREEN BUILDING FOCUS GROUP

WORKSHOP #1

October 10, 2017 – Agenda Item 1

Alignment with the California Building Standards Commission’s (CBSC)

2016 Intervening Code revisions

- DSA proposes amend subsections of Section 5.303 to co-adopt modifications made by CBSC during the 2016 Intervening Code Cycle which now aligns with California Energy Commission’s (CEC) Title 21.  (Below are the revisions with strikeout and underline.)

Shower Heads

5.303.3.2 Urinals….

5.303.3.3 Showerheads.

5.303.3.3.1 Single showerhead. Showerheads shall have a maximum flow rate of not more than 2.0 1.8 gallons per minute at 80 psi. Showerheads shall be certified to the performance criteria of the U.S. EPA WaterSense Specification for Showerheads.

5.303.3.3.2 Multiple showerheads serving one shower. When a shower is served by more than one showerhead, the combined flow rate of all showerheads and/or other shower outlets controlled by a single valve shall not exceed 2.0 1.8 gallons per minute at 80 psi, or the shower shall be designed to allow only one shower outlet to be in operation at a time.

Note: A hand-held shower shall be considered a showerhead.

5.303.3.4 Faucets and fountains…

- DSA proposes to amend Section 5.106.8 to co-adopt modifications made by CBSC during the 2016 Intervening Code Cycle. (Below are the revisions with strikeout and underline.)

5.106.8 Light Pollution Reduction

SECTION 5.106
SITE DEVELOPMENT

…

5.106.8 Light pollution reduction. [N] Outdoor lighting systems shall be designed and installed to comply with the following:

- The minimum requirements in the California Energy Code for Lighting Zones 01  -4 as defined in Chapter 10,  Section 10-114 of the California Administrative Code; and

- Backlight, Uplight and Glare (BUG) ratings as defined in IES TM-15-11(shown in TABLE A-1 in chapter 8);

- Uplight and Glare ratings as defined in California Energy Code (shown in TABLES 130.2-A and 130.2B in chapter 8) and

4. Allowable BUG ratings not exceeding those shown in Table 5.106.8 [N], or

Comply with a local ordinance lawfully enacted pursuant to Section 101.7, whichever is more stringent.

Exceptions: [N]

- Luminaires that qualify as exceptions in Section 140.7 of the California Energy Code.

Emergency lighting.

- Building facade meeting the requirements in Table 140.7-B of the California Energy Code, Part 6.

- Custom lighting features as allowed by the local enforcing agency, as permitted by Section 101.8 Alternate materials, designs and methods of construction

Note: [N] See also California Building Code, Chapter 12, Section 1205.6 for college campus lighting requirements for parking facilities and walkways.

TABLE 5.106.8 [N]
MAXIMUM ALLOWABLE BACKLIGHT, UPLIGHT AND GLARE (BUG) RATINGS1,2

| ALLOWABLE RATING | LIGHTING ZONE LZ0 | LIGHTING ZONE LZ1 | LIGHTING ZONE LZ2 | LIGHTING ZONE LZ3 | LIGHTING ZONE LZ4 |
| --- | --- | --- | --- | --- | --- |
| Maximum Allowable Backlight Rating3 ( B) |  |  |  |  |  |
| Luminaire greater than 2 mounting heights (MH) from property line | N/A | No Limit | No Limit | No Limit | No Limit |
| Luminaire back hemisphere is 1 – 2 MH from property line | N/A | B2 | B3 | B4 | B4 |
| Luminaire back hemisphere is 0.5 – 1 MH from property line | N/A | B1 | B2 | B3 | B3 |
| Luminaire back hemisphere is less than 0.5 MH from property line | N/A | B0 | B0 | B1 | B2 |
| Maximum Allowable Uplight Rating (U) |  |  |  |  |  |
| For area lighting4 | N/A | U0 | U0 | U0 | U0 |
| For all other outdoor lighting, including decorative luminaires | N/A | U1 | U2 | U3 | U4 |
| Maximum Allowable Glare Rating5 (G) |  |  |  |  |  |
| Luminaire greater than 2 MH from property line | N/A | G1 | G2 | G3 | G4 |
| Luminaire front hemisphere is 1 – 2 MH from property line | N/A | G0 | G1 | G1 | G2 |
| Luminaire front hemisphere is 0.5 – 1 MH from property line | N/A | G0 | G0 | G1 | G1 |
| Luminaire back front hemisphere is less than 0.5 MH from property line | N/A | G0 | G0 | G0 | G1 |

- IESNA Lighting Zones 0 and 5 are not applicable; refer to Lighting Zones as defined in the California Energy Code and Chapter 10 of the California Administrative Code.

- ...

Note: Refer to Chapter 8 (Compliance Forms, Worksheets and Reference Material) for IES TM-15-11 Table A-1, California Energy Code Tables 130.2-A and 130.2-B.

. . .